IO-VNBD Dataset – Column Reference Guide

Smartphone (S) and Vehicle (V) columns for Intelligent Dead Reckoning

A practical reference explaining what each dataset column represents and how it can be used in an AI/ML-based GNSS-denied navigation system.

1. Smartphone (S) — 24 Columns

| # | Column | What it means | Use in project |
| --- | --- | --- | --- |
| 1 | GPS LATITUDE (degrees) | North/south geographic position. | Reference position; convert to local X/Y. |
| 2 | GPS LONGITUDE (degrees) | East/west geographic position. | Reference position; convert to local X/Y. |
| 3 | GPS ALTITUDE (m) | Height above reference level. | 3D position; usually less important for 2D vehicle navigation. |
| 4 | GPS SPEED (Kmh) | Speed reported by smartphone GPS. | Reference/validation velocity. |
| 5 | GPS ACCURACY (m) | Estimated GPS position uncertainty. | GPS reliability; filtering/weighting. |
| 6 | GPS ORIENTATION (°) | Direction reported by GPS/device. | Heading/orientation validation. |
| 7 | GPS SATELLITES IN RANGE | Number of visible satellites. | GPS quality indicator. |
| 8 | TIME SINCE START (ms) | Elapsed time from recording start. | Synchronization and Δt calculation. |
| 9 | DATE (YYYY-MO-DD HH-MI-SS_SSS) | Measurement timestamp. | Time alignment/synchronization. |
| 10 | ACCELEROMETER X (m/s²) | Acceleration along sensor X. | Major ML input. |
| 11 | ACCELEROMETER Y (m/s²) | Acceleration along sensor Y. | Major ML input. |
| 12 | ACCELEROMETER Z (m/s²) | Acceleration along sensor Z. | Major ML input. |
| 13 | GRAVITY X (m/s²) | Estimated gravitational component on X. | Gravity compensation/orientation. |
| 14 | GRAVITY Y (m/s²) | Estimated gravitational component on Y. | Gravity compensation/orientation. |
| 15 | GRAVITY Z (m/s²) | Estimated gravitational component on Z. | Gravity compensation/orientation. |
| 16 | GYROSCOPE Yaw (rad/s) | Angular velocity around yaw. | Major ML input; turning/rotation. |
| 17 | GYROSCOPE Pitch (rad/s) | Angular velocity around pitch. | ML input; body motion. |
| 18 | GYROSCOPE Roll (rad/s) | Angular velocity around roll. | ML input; body motion. |
| 19 | MAGNETIC FIELD X (μT) | Magnetic field along X. | Heading/orientation estimation. |
| 20 | MAGNETIC FIELD Y (μT) | Magnetic field along Y. | Heading/orientation estimation. |
| 21 | MAGNETIC FIELD Z (μT) | Magnetic field along Z. | Heading/orientation estimation. |
| 22 | ORIENTATION (Yaw) (°) | Device heading around vertical axis. | Heading/orientation. |
| 23 | ORIENTATION (Pitch) (°) | Device forward/backward tilt. | Coordinate transformation/orientation. |
| 24 | ORIENTATION (Roll) (°) | Device sideways tilt. | Coordinate transformation/orientation. |

Note: some smartphone subsets use GYROSCOPE X/Y/Z naming instead of Yaw/Pitch/Roll. Do not rename these as equivalent axes without verifying the sensor convention.

2. Vehicle (V) — 29 Columns

| # | Column | What it means | Use in project |
| --- | --- | --- | --- |
| 1 | No. GPS satellites | Number of visible/used satellites. | GPS quality. |
| 2 | Time since start of day | Vehicle time reference. | Timestamp/synchronization. |
| 3 | Latitude | Vehicle geographic latitude. | Reference trajectory. |
| 4 | Longitude | Vehicle geographic longitude. | Reference trajectory. |
| 5 | Velocity | Vehicle speed/velocity. | Reference velocity / possible target. |
| 6 | Heading | Direction of vehicle travel. | Reference heading. |
| 7 | Height | Vehicle elevation. | 3D/reference position. |
| 8 | Vertical velocity | Vertical vehicle velocity. | Vertical motion. |
| 9 | Sample period | Time between measurements. | Integration and synchronization. |
| 10 | Steering angle | Amount steering is turned. | Turning behaviour. |
| 11 | Wheel speed FL | Front-left wheel speed. | Vehicle motion/velocity. |
| 12 | Wheel speed FR | Front-right wheel speed. | Vehicle motion/velocity. |
| 13 | Wheel speed RL | Rear-left wheel speed. | Vehicle motion/velocity. |
| 14 | Wheel speed RR | Rear-right wheel speed. | Vehicle motion/velocity. |
| 15 | Yaw rate | Vehicle rotation rate about vertical axis. | Turning/heading estimation. |
| 16 | Indicated vehicle speed | Vehicle internally indicated speed. | Velocity reference. |
| 17 | Indicated longitudinal acceleration | Forward/backward acceleration. | Compare with smartphone acceleration. |
| 18 | Indicated lateral acceleration | Sideways acceleration. | Turning/cornering reference. |
| 19 | Handbrake | Handbrake status. | Vehicle state. |
| 20 | Gear requested | Requested gear. | Driving-state feature. |
| 21 | Gear | Current gear. | Driving-state feature. |
| 22 | Engine speed | Engine RPM. | Vehicle operating condition. |
| 23 | Coolant temperature | Engine coolant temperature. | Vehicle operating condition; usually not navigation-critical. |
| 24 | Clutch | Clutch state/measurement. | Vehicle operating condition. |
| 25 | Brake pressure | Brake system pressure. | Braking behaviour. |
| 26 | Brake position | Brake pedal/position. | Braking behaviour. |
| 27 | Battery voltage | Vehicle electrical voltage. | System health; usually not navigation-critical. |
| 28 | Air temperature | Measured air temperature. | Environmental/vehicle condition. |
| 29 | Accelerator pedal position | Accelerator amount. | Acceleration/driving behaviour. |

3. Recommended ML role

Main smartphone inputs: Accelerometer, gyroscope, gravity, magnetic field and orientation. These describe motion and orientation available from the phone.

Reference/targets: GPS latitude/longitude/speed and vehicle latitude/longitude, velocity, heading, acceleration and yaw rate can support supervised training and evaluation. During simulated GNSS blackout, GPS should not be supplied as an inference input.

Preprocessing/control: Time since start, date, sample period, GPS accuracy and satellite count support synchronization, Δt calculation, quality checks and sequence construction.

Usually not first-model inputs: Engine speed, coolant temperature, battery voltage, air temperature, clutch, gear, handbrake, brake and accelerator describe vehicle state but are not core smartphone dead-reckoning signals.

4. Jury must-know columns

Smartphone: Accelerometer X/Y/Z, Gyroscope, Gravity X/Y/Z, Magnetometer X/Y/Z, Orientation, GPS Lat/Lon, GPS Speed, GPS Accuracy and timestamps.
Vehicle: Velocity, Heading, Wheel Speeds, Yaw Rate, Longitudinal Acceleration and Lateral Acceleration.

Key distinction: S = smartphone-side sensor data; V = vehicle/CAN-side data. The smartphone can be the main inference input, while synchronized vehicle/GPS measurements provide reference information for supervised training and evaluation.